# Van’s Aircraft RV-6A

# Pilot’s Operating Handbook

N596S

Bruce Patton

INDEX

General........................................................... 3

Limitations...................................................... 6

Normal Operating Procedures ........................ 8

Emergency Operating Procedures ................. 15

Performance Specifications .......................... 18

Weight and Balance ......................................19

Equipment List............................................. 22

Service/maintenance information ................. 25

PERFORMANCE – SPECIFICATIONS

SPAN: ………………………………………….……………23’ 0”

LENGTH: …………………………………..……………….19’ 9”

HEIGHT:  …………………………………………………….6’ 8”

SPEED:

Maximum at Sea Level ………………………..TBD Knots

Cruise, 75% Power at 8,000 Ft ………………..TBD Knots

RANGE (includes 3 gal. for taxi, takeoff & climb):

75% @ 8000’ , no reserve .………………..…….TBD nm

55% @ 8000’  no reserve ….……………......…..TBD nm

75% @ 8000’, one hour (8 gal) reserve ……...…..TBD nm

55% @ 8000’, one hour (8 gal) reserve …...……..TBD nm

RATE OF CLIMB AT SEA LEVEL …………..…….….TBD  FPM

SERVICE CEILING ………………………………...…….TBD FT

TAKEOFF PERFORMANCE: ……………….……….……TBD Ft

LANDING PERFORMANCE: ………………………..……TBD Ft

STALL SPEED (CAS):

Flaps Up, Power Off ………………………..…TBD Knots	Flaps Down, Power Off ………………….……TBD Knots

MAXIMUM WEIGHT (Normal Category): ….….……….1900 Lbs

EMPTY WEIGHT …………………………….…….…….1148 Lbs

MAXIMUM USEFUL LOAD: ……………………...……..752 Lbs

BAGGAGE ALLOWANCE …………………………..…… 75 Lbs

WING LOADING (Pounds/ Sq. Ft) ……………........….… 15 PSF

POWER LOADING (Pounds/ HP) …….......…… ……… 10.5 PSF

FUEL CAPACITY .....………………………………………38 Gal Total

Type ……………………………………………….100 LL

OIL CAPACITY …………………………………………….	8 Qts

ENGINE: Lycoming …………..……………..………..O-360-A1A

PROPELLER: Hartzell ……….………………….   	HC-C2YK-BF

##### Engine Information

Model: ……..	Lycoming O-360 A1A

HP: …………	180

Fuel: ………..	100LL

………..

Oil Filter: …..	Champion CH48110

Spark Plugs …

Alternator Belt…  Gates 7365 9.5/10 MM X 945 MM

Vacuum Filter … D9-18-1 Airborne

Air Filter … K & N

OIL:  Avg	MIL-L-6082	Ashless Dispersant

Ambient Air	   Grades		     Grades

All Temps.			      SAE 15W50 or 20W50

Above 80F	SAE 60		      SAE 60

Above 60F	SAE 50		      SAE 40 or SAE 50

30 – 90F		SAE 40		      SAE 40

0-70F		SAE 20		      SAE 30 or SAE 40

###### Below 10F		SAE 20		      SAE 30 or 20W30

###### Oil Sump

###### Capacity …….. 8 US Quarts

###### Minimum Safe Quantity ...2 US Quarts

###### Operating Conditions:

###### Oil Inlet Temp:		180 deg F desired, 245 deg F Maximum

###### Oil Pressure:		95psi max; 	 55 psi min            25 psi idle

###### Fuel Pressure:		 8 psi max;	 0.5 psi min;	 3.0 psi desired

Cyl. Head Temp	 500 deg F Max,  400 deg F desired max, 150 deg F min

| Altitude, ft. | Percent Power | Full throttle HP |
| --- | --- | --- |
| 0 | 100 | 180 |
| 2,000 | 93 | 168 |
| 4,000 | 88 | 158 |
| 6,000 | 82 | 147 |
| 8,000 | 76 | 137 |
| 10,000 | 71 | 127 |
| 12,000 | 66 | 118 |
| 14,000 | 61 | 110 |
| 16,000 | 57 | 102 |
| 18,000 | 52 | 94 |

LIMITATIONS

AIRSPEED LIMITATIONS

|  | SPEED | KIAS | REMARKS |
| --- | --- | --- | --- |
| VNE | Never Exceed Speed | 182 Kts | Do not exceed in any operations. |
| #### VNO | Maximum Structural Cruising Speed | 156 Kts | Exceed this speed only in smooth air. |
| VA | Maneuvering Speed | 115 Kts | No full control movements. Full elevator deflection will result in a 6g load |
| VFE | Maximum Flap Extended Speed | 90 Kts | Do not exceed with flaps down |
| Vy | Best Rate of Climb | #### TBD Kts |  |
| Vx | Best Angle of Climb | TBD Kts |  |
| Vs | Stall Speed Clean | TBD Kts |  |
| Vso | Stall Speed Landing Configuration | TBD Kts |  |

AIRSPEED INDICATOR MARKINGS

| MARKING | VALUE OR RANGE | SIGNIFICANCE |
| --- | --- | --- |
| White Arc | 43 – 90 Kts | Full Flap Operating Range. |
| Green Arc | 47 – 156 Kts | Normal Operating Range. Lower limit is Vs. Upper limit is maximum structural cruising speed |
| Yellow Arc | 156 – 182 Kts | Operations with caution and only in smooth air. |
| Red Line | 182 Kts | Maximum speed for all operations |

LOADING LIMITATIONS

With full tanks and 75 lb. Baggage, maximum pilot plus passenger weight is 470 lb.   The aircraft is limited by Gross weight and the CG is forward of the aft limit for zero fuel conditions.  With no baggage, pilot and passenger minimum weight is less than 160 Lbs., fuel 0 to 34 gallons..

UTILITY CATEGORY

##### AEROBATIC INFORMATION

Weight Limitation – 1375 Pounds

Recommended Entry Speeds:

Loops, Horizontal Eights ….	120 – 165 Kts

Immelman Turns …………..	130 – 165 Kts

Aileron Rolls, Barrel Rolls ..	105 – 165 Kts

Snap Rolls………………….. 70  –   95 Kts

Vertical Rolls ………………	155 –165 Kts

Split-S ……………………… 85  -  95 Kts

NORMAL PROCEDURES

##### PREFLIGHT INSPECTION

1.	CABIN

- Documentation – CORRECT AND AVAILABLE

- Aeronautical Charts – CURRENT

- Seat Belt Securing Control Stick --  RELEASE

- Key - ON DASH

- Avionics -- OFF

- Bat  Switch – ON

- Flaps - DOWN

- Fuel Quantity -- CHECK QUANTITY

- Fuel Boost Pp -  ON

- Bat switch - OFF

2. EMPENNAGE

- Static source right side - CLEAR

- Tail Tie-Down -- DISCONNECT

- Control Surfaces -- CHECK freedom of movement and security

- Static source left side - CLEAR

3. LEFT WING

- Flap -- CHECK security

- Aileron -- CHECK freedom of movement and security

- Landing and Taxi  Light -- CHECK condition

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap – SECURE

- Wing Tie-Down – DISCONNECT AND REMOVE RING

- Pitot Tube Cover -- REMOVE and check for blockage

- Main Wheel Tire -- CHECK for proper inflation

- Chock – REMOVE

- Wing Tank – SUMP

- Bat switch - ON

- Gascolator – DRAIN SUMP

- Bat switch  – OFF

5. NOSE

- Engine Oil Level -- CHECK, do not operate with less than 5 quarts

- Propeller and Spinner -- CHECK for nicks and security

- Cowl Hinge Pins – CHECK  (4) for security

- Air Inlet -- CHECK for restrictions

- Nose Wheel Tire -- CHECK for proper inflation

- Chock -- REMOVE

- Fuel Tank Vents -- CHECK for blockage

6. RIGHT WING

- Left Wing Tank – SUMP

- Wing Tie-Down – DISCONNECT AND REMOVE RING

- Main Wheel Tire -- CHECK for proper inflation

- Chock -- REMOVE

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap -- SECURE

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

### STARTING ENGINE

- Bat and Alt. - ON

- ATIS or Clearance

- Alt - ON

- Fuel - ON

- Carb Heat - COLD

- Mixture - RICH

- Prop - HIGH RPM

- Throttle - ¼”

- Fuel Boost - ON

- Prime - 2-3 SEC (If red)

- Brakes Set

- Clear Prop and Start

- Verify Oil Pressure

### TAXI

- Fuel Boost - OFF

- Seats and Belts - CHECK

- Transponder - ON

- Radio - ON and SET

- Flaps - UP

- Check Brakes

- Ground Control - TALK

- Verify DG to Compass

### RUN-UP

- Brakes - SET

- Instruments

- Airspeed OK

- Horizon OK

- Altimeter -VERIFY

- Set DG to Compass

- Fuel - FULLEST

- Elevator trim - SET

- Aileron trim - SET

- Prop - HIGH RPM

- Mixture - RICH

- Throttle 1700 RPM

- Engine Gages - Check

- Mags 125 drop, 50 diff

- Suction

- Carb Heat - RPM DROP

- Cycle Prop

- Alternator OFF- ON ALARM

- Throttle - IDLE

- Fuel Boost - ON

- Check Controls

- Lights - AS REQD.

- Canopy - LOCKED

### TAKEOFF

- Radio - SET

- Transponder - ALT

- Flaps - AS REQD.

- DG to Runway Hdg.

- Carb Heat - COLD

- Prop rpm - HIGH

- Communicate

- Throttle - FULL

- Oil Pressure - Check

- Rotate at 55 K

- Climb at 100 K

### CLIMB

- At 1000’ AGL

- Flaps - UP

- Fuel Boost - OFF

- Check Fuel Pressure

- Prop 2500 RPM

- Lean if REQD.

### CRUISE

- Throttle 23”

- Prop 2300 RPM

- Trim

- Lean to 100 Rich

- Set AUTOTRAC

### INCREASE SPEED CRUISE

- AUTOTRAC OFF

- Mixture RICH

- Prop INCREASE RPM

- Throttle INCREASE

- Lean to 100 Rich

- Set AUTOTRAC

### DESCENT

- AUTOTRAC OFF

- Mixture Rich

- Carb Heat if Reqd

- Throttle as reqd

- Prop as reqd

- Lean if reqd.

APPROACH

- Mixture Rich

- Carb heat if Reqd

- Throttle as Reqd.

- Prop as Reqd.

- Slow to 100K

- Fuel Tank Select

- Boost pump on

- Approach frequency

- Tower frequency

LANDING

- Mixture Rich

- Carb Heart

- Throttle to 1700

- Hold at 70K

- Flaps +10

- Descent

- Flaps as Reqd

- Short Final at 65K

- Land

- Flaps UP

- Fuel Boost OFF

SHUTDOWN

- Radio OFF

- Transponder OFF

- Mixture LEAN

- Alternator OFF

- Bat OFF

- Key - ON DASH

#### SHORT FIELD TAKEOFF

- Wing Flaps – 10 Deg

- Carb Heat – COLD

- Prop – HIGH RPM

- Brakes – APPLY

- Throttle – FULL OPEN

- Mixture – RICH (above 3000’ lean to obtain max RPM)

- Brakes – RELEASE

- Climb Speed – 80 KIAS (Vy)

##### ENROUTE CLIMB

- Airspeed – 110 - 130 KIAS

- Throttle – 25 in Hg, or full throttle

- Prop – 2500 RPM

- Boost Pump – OFF at 1000 feet AGL

- Fuel Pressure – CHECK

- Mixture – LEAN above 5000’

#### SECURING AIRCRAFT

- Wheel Chocks – INSTALL

- Wing & Tail Tie-Down – INSTALL AS REQUIRED

- Pitot Tube Cover – INSTALL AS REQUIRED

- Cockpit – CLEAN AND SECURE

- Ignition Key – REMOVED

- Master and Electrical Switches – OFF

- Engine Monitor Fuel Quantity – RESET AFTER REFUELING

EMERGENCY PROCEDURES

ENGINE FAILURES

ENGINE FAILURE DURING TAKEOFF RUN

- Throttle –IDLE

- Brakes – APPLY

- Wing Flaps – RETRACT

- Mixture – IDLE CUT-OFF

- Ignition Switch – OFF

- Master Switch – OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed – 70 KIAS

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – AS REQUIRED

- Master Switch – OFF

ENGINE FAILURE DURING FLIGHT

- Airspeed –80 Knots

- Boost Pump – ON

- Fuel Selector – SWITCH TANKS

- Carb Heat – ON

- Mixture – RICH

- Primer - ON to run

- Ignition Switch – BOTH, LEFT, RIGHT

- Transponder – 7700

FIRE DURING START ON GROUND

- Cranking – CONTINUE, to get a start which would suck the flames and accumulated fuel through the carb and into the engine.
If engine starts:

- Power – 1700 RPM for a few minutes

- Engine – SHUTDOWN and inspect for damage

- If engine fails to start:

- Throttle – FULL OPEN

- Mixture – IDLE CUT-OFF

- Cranking – CONTINUE

- Fire Extinguisher – OBTAIN

- Engine – SECURE

ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Master Switch – OFF

- Cabin Heat and Air – OFF

ELECTRICAL FIRE IN FLIGHT

- Master Switch – OFF

- Avionics – OFF

- Vents/ Cabin Air/ Heat – CLOSED

- Fire Extinguisher – ACTIVATE

- Alt Vital - ON if fire extinguished

CABIN FIRE

- Master Switch – OFF

- Vents/ Cabin Heat – CLOSED

- Fire Extinguisher – ACTIVATE (if available)

WING FIRE

- Nav Lights – OFF

- Landing Light – OFF

ELECTRICAL / ALTERNATOR FAILURE

- Avionics –OFF

- Master Switch – OFF

- Reset Alt Breaker

- Check Alt fuse (non-vital)

- IF ALTERNATOR IS STILL OFF-LINE:

- Master Switch – OFF

- Alt Vital feed - ON

- Avionics – ON as required

- Electrical Equipment – ON, as required

- Flight – TERMINATE as soon as practical,  aircraft is on battery reserves only.

PERFORMANCE SPECIFICATIONS (TBD)

Cruise Performance at 8,000’:

| KTAS | RPM | MAP | Fuel Flow | % Power |
| --- | --- | --- | --- | --- |
| 167 | 2400 | 23” | 8.4 GPH | 75% |
| 157 | 2200 | 23” | 7.4 GPH | 65% |
| 151 | 2200 | 21” | 6.3 GPH | 55% |

No Wind Range at 8,000’: (ESTIMATED TBD in flight test program)

- All range calculations include 3 gal. for engine start, taxi, takeoff and climb.

- Engine is leaned for best economy.

One hour (8 gal.) reserve:

75% Power …………………………………..	540 nm

65% Power …………………………………..	575 nm

55% Power …………………………………..	650 nm

No Reserve:

75% Power …………………………………..	695 nm

65% Power …………………………………..	745 nm

55% Power …………………………………..	840 nm

##### WEIGHT AND BALANCE DATA

Make: Bruce Patton		Model:  RV-6A

Serial: 23042			Registration: N596S

Maximum Weights:

Aerobatic Category …………………	1475 Lbs

Utility Category …………………….	1650 Lbs

Normal Category ……………………	1900 Lbs

Datum 60 inches forward of wing leading edge (L.E.)

Design C.G. Range = 	15% to 29% of wing chord

8.7” to 16.8” from L.E.

68.7” to 76.8” aft of Datum

Wing L.E. = 60 inches aft of datum

Main wheel right =  84.5”  aft of datum

Main wheel left = 84.5”   aft of datum

Nose wheel = 28.25”   aft of datum

Fuel ……………70” aft of datum

Pilot and Passenger …87.4” aft of datum

Baggage ....117”  aft of datum	
Aircraft weighed empty in level flight attitude.

(Includes 8 qts. of oil, no useable fuel)

Weight		Arm		Moment

Right Wheel		394		84.5		33293

Left Wheel		410		84.5		34645

Nose Wheel		344		28.25		9718

Total			1148				77656

Empty CG = 67.64” aft of datum

##### WEIGHT AND BALANCE CALCULATION

Sample:

Weight		Arm		Moment

Aircraft	1148			`	77656

Fuel (6lbs/gal)			70

Pilot 				87.4

Passanger	______	87.4		______

Baggage			117

Total

CG = Total Moment / Total Weight

CG = 		 in aft of datum

CG Range = 68.7 to 76.8 in aft of datum

Calculations for Limitations Statement on Weight and Balance

|  | Max Pilot/passenger, baggage and fuel case |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weight | Arm | Moment |  |
| Aircraft | 1148 |  | 77656 |  |
| Fuel enter below |  |  |  |  |
| 34 | 204 | 70 | 14280 |  |
| Pilot & passenger | 470 | 87.4 | 34960 |  |
| Baggage | 75 | 117 | 8775 |  |
| Total | 1893 |  | 141789 |  |
| C.G = | 74.7” | Aft of datum |  | (68.7 to 76.8) |
| C.G. is |  | Good |  |  |
| Gross Weight is |  | Good. |  | (1900 Max) |

|  | Max Pilot/passenger, baggage with no fuel case |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Weight | Arm | Moment |  |
| Aircraft | 1148 |  | 77656 |  |
| Fuel enter below |  |  |  |  |
| 0 | 0 | 70 | 0 |  |
| Pilot& passanger | 470 | 87.4 | 34960 |  |
| Baggage | 75 | 117 | 8775 |  |
| Total | 1693 |  | 127509 |  |
| C.G = | 75.3” | Aft of datum |  | (68.7 to 76.8) |
| C.G. is |  | Good |  |  |
| Gross Weight is |  | Good. |  | (1900 Max) |
|  | Max pilot/passenger, no baggage max. fuel case |  |  |  |  |  |  |
|  | Weight | Arm | Moment |  |
| Aircraft | 1148 |  | 77656 |  |
| Fuel enter below |  |  |  |  |
| 34 | 204 | 70 | 14280 |  |
| Pilot & passanger | 470 | 87.4 | 34960 |  |
| Baggage | 0 | 117 | 0 |  |
| Total | 1822 |  | 133014 |  |
| C.G = | 73.0” | Aft of datum |  | (68.7 to 76.8) |
| C.G. is |  | Good |  |  |
| Gross Weight is |  | Good. |  | (1900 Max) |
|  | Min. pilot/passenger, no baggage no fuel case |  |  |  |  |  |  |
|  | Weight | Arm | Moment |  |
| Aircraft | 1148 |  | 77656 |  |
| Fuel enter below |  |  |  |  |
| 0 | 0 | 70 | 0 |  |
| Pilot & passenger | 160 | 87.4 | 13984 |  |
| Passenger | 0 | 87.4 | 0 |  |
| Baggage | 0 | 117 | 0 |  |
| Total | 1308 |  | 91640 |  |
| C.G = | 70.2” | Aft of datum |  | (68.7 to 76.8) |
| C.G. is |  | Good |  |  |
| Gross Weight is |  | Good. |  | (1900 Max) |

EQUIPMENT LIST

| Reqd* |  | Equipment Description |  | Manufacture |
| --- | --- | --- | --- | --- |
|  | Powerplant and Accessories |  |  |  |
|  | Engine, including starter |  | Lycoming |  |
| R | Alternator, 35 Amp |  | B&D |  |
|  | Oil Cooler |  |  |  |
|  | Oil Filter, spin on |  | Champion CH48110 |  |
|  | Propeller, Constant speed |  | Hartzcll |  |
|  | Vacuum Pump, CCW rotation |  |  |  |
|  | Vacuum regulator |  |  |  |
|  | Vacuum filter |  |  |  |
|  | Oil Separator |  | ACS |  |
|  | ELECTRICAL AND LIGHTS |  |  |  |
| R | Voltage Regulator |  | B&D |  |
| R | Battery, 12 V |  |  |  |
| R-N | Instrument Lights |  |  |  |
|  | Cabin light |  |  |  |
|  | Baggage light |  |  |  |
|  | Light Dimmer |  | Homebuilt |  |
|  | Taxi light |  | 75 W Halogen |  |
|  | Landing light |  | 75 W Halogen |  |
| R-N | Navigation lights, wingtip |  | Whelen |  |
| R-N | Strobe lights, wingtip |  | Whelen |  |
|  | Fuel selector indicating lights |  | HB |  |
|  | EC-100 Checklist and enunciator |  | Vision Microsystems |  |
|  | Intercom system |  |  |  |
|  | Ignition switch |  |  |  |
|  | INSTRUMENTS |  |  |  |
| R | Airspeed, true |  |  |  |
|  | Attitude gyro, Vacuum |  | RC Allen |  |
| R | Altimeter, sensitive |  |  |  |
|  | Turn Coordinator, electric |  |  |  |
|  | Directional Gyro, Vacuum |  |  |  |
|  | Vertical Speed indicator |  |  |  |
|  | Suction gage |  |  |  |
| R | Fuel gages, left and right |  |  |  |
|  | Elevator trim position |  |  |  |
| R | Compass, magnetic |  |  |  |
|  | Aileron Trim position |  |  |  |
|  | VM 1000 Engine Instrument including |  |  |  |
|  | Manifold pressure |  |  |  |
| R | Engine RPM |  |  |  |
|  | EGT, all cylinders |  |  |  |
|  | Head Temps, all cylinders |  |  |  |
|  | Fuel pressure |  |  |  |
| R | Oil Pressure |  |  |  |
|  | Voltage |  |  |  |
|  | Fuel computer, flow |  |  |  |
| R | Oil temperature |  |  |  |
| R | Amperage |  |  |  |
|  | Carb./outside air temp. |  |  |  |
|  | Hobbs meter |  |  |  |
|  | RADIO/ELECTRONICS |  |  |  |
|  | Terra 760D radio |  | Terra |  |
|  | Terra 250D Transponder |  | Terra |  |
|  | Terra altitude encoder |  | Terra |  |
|  | ELT |  | Ameri-King AK-450 |  |
|  | CABIN |  |  |  |
| R | Seat and shoulder belts, pilot |  |  |  |
| R | Seat and shoulder belts, passenger |  |  |  |
| R | Passenger Warning placard |  |  |  |
| R | Day/Night limitations placard |  |  |  |

R ---- Required per FARs

R-N - Required per FARs for night operation

##### SERVICE/MAINTENANCE

##### ANNUAL CONDITION  INSPECTION REPORT

HOURS:

MAKE: RV-6A				SERIAL #:23042

REGISTRATION #:  N596S		DATE:

DESCRIPTION : 				COMMENTS:

A.        PROPELLER GROUP

- Inspect spinner and back plate....

- Inspect blades for nicks and cracks....

- Check for grease and oil leaks....

- Lubricate propeller per lubrication chart....

- Check spinner mounting brackets....

- Check propeller mounting bolts and safety....

- Inspect hub parts for cracks and corrosion....

B.        ENGINE GROUP

CAUTION:        ground mags before working on engine

- Remove engine cowl....

- Clean and check cowling for cracks, distortion and loose or missing fasteners....

- Drain oil sump....

- Change oil filter at oil change....

- Check oil temp. sender for leaks and security....

- Check oil lines and fittings for leaks, chafing, security, dents and cracks....

- Clean and check oil radiator cooling fins....

- Fill engine with oil per lubrication chart...

- Clean engine....

- Check condition of spark plugs adjust gap....

- Check ignition harness and insulators....

- Check magneto points for proper clearance maintain at .018 +- .006....

- Check magneto for oil seal leaks....

- Check breaker felt for proper lubrication....

- Check distributor block for cracks, burned areas or corrosion....

- Check magnetos to engine timing....

- Remove air filter and clean....

- Drain carburetor and clean inlet line fuel strainer....

- Check condition of carburetor heat air door	and box....

- Check intake seal for leaks and clamps for tightness....

- Clean screens in fuel pump....

- Clean and check gascolator....

- Inspect condition of fuel lines....

- Check fuel system for leaks.....

- Check electric fuel pump for operation.....

- Check engine controls throttle, carb heat, mixture, and prop....

- Inspect exhaust stacks, connections, gaskets....

- Inspect heat exchange and baffles....

- Check breather tube for obstructions and security....

- Check crankcase for cracks, leaks, security of bolts....

- Check engine mounts for cracks and loose mountings.......

- Check all engine baffles....

- Check firewall seals....

- Check condition and tension of alternator and drive belt....

- Check condition of starter....

- Check fluid in brake reservoir....

- Lubricate all controls....

- Reinstall engine cowl....

C.          CABIN GROUP

- Inspect canopy and windows....

- Check   upholstery for tears....

- Check seats, seat belts, shoulder straps....

- Check trim operation...

- Check rudder pedals....

- Check control column, systems and connection.....

- Check landing, nav, cabin, and instrument lights...

- Check instrument, lines and attachments....

- Check gyro operation, and electric turn and bank....

- Check or replace gyro air filter....

- Clean or replace vacuum regulator filter....

- Check altimeter....

- Check operation of fuel selector valve....

- Check operation of engine primer....

- Check condition of heater controls....

- Check condition and operation air vents....

- FUSELAGE AND EMPENNAGE GROUP

- Remove inspection plates and panels...

- Check battery box, battery and cables...

- Check electronic installations...

- Replace ELT batteries

- Check bulkheads and stringers for damage

- .Check antenna mounts and wiring....

- Check fuel lines and valves....

- Check vertical fin and rudder surfaces....

- Check rudder horn and attachment...

- Check vertical fin attachments....

- Check rudder bolts for wear...

- Check horizontal stabilizer and elevators...

- Check horizontal stabilizer attachment...

- Check elevator horn.

- Check elevator bolts for wear...

- Check elevator bell cranks and controls....

- Lubricate all bearings as needed.....

- Reinstall inspection plates and panels....

- WING GROUP

- Remove inspection plates...

- Check surfaces and tip for damage and loose rivets and condition of walkways....

- Check aileron mounts and attachments....

- Check aileron bellcrank and control tubes....

- Check flaps and attachment for damages....

- Check all wing attachment bolts....

- Check fuel tanks and lines for leaks and....

- Check fuel tank vents....

- Reinstall inspection plates....

- LANDING GEAR GROUP

- NDT Nose strut per Vans Aircraft Service recommendation

- Check nose gear travel and damper tightness....

- Check main gear attachments...

- Check tires for cuts and wear....

- Remove wheel, clean, and repack bearings...

- Check wheels for cracks, corrosion and broken bolts....

- Check tire pressure....

- Check brake lining and disc....

- Check brake lines....

- OPERATIONAL INSPECTION

- Check fuel pump and fuel tank selector....

- Check fuel pressure....

- Check oil pressure and temperature....

- Check alternator output....

- Check manifold pressure....

- Check alternate air....

- Check vacuum gauge....

- Check gyros for noise and roughness....

- Check Cabin heat operation....

- Check magneto switch operation....

- Check magneto RPM variation....

- Check throttle and mixture operation....

- Check propeller smoothness....

- Check electronic equipment operation....

G 	Known problem areas:

- Fretting corrosion at gear leg fairing intersection

- Corrosion of exposed rod end bearings

- Rudder contacting elevator roots in flight (dynamic clearance less than static Clearance!)

- Chafing brake lines on gear leg fairing and beneath wheel pant

NOTES

MAINTENANCE PROCEDURES

Removal of Instrument Panel

Tools: 4-40 allen, 9/16, 5/8 wrench, 3/8 socket, Phillips, short straight and normal length screwdriver, Band-Aids.

- Under Panel

  - Disconnect battery

- Outside panel

  - Remove Radio

  - Remove Transponder

  - Remove VM1000 display and unplug (#4-40 allen)

  - Unplug flat cable to Carb/outside air gage

- Under Panel

  - Left Cabin air at panel

  - Disconnect Static line red (9/16 & 5/8 wrench)

  - Disconnect Pitot  line white

  - Disconnect D.G electrical plug

  - Disconnect flat plug to EC100

  - Disconnect power plug to EC100

  - Disconnect plug to fuel gages/LEDs.

  - Disconnect pitot tube at rear of VSI that runs to encoder (9/16 wrench)

  - Remove screw holding gyro air filter to panel (3/8 socket and Phillips screwdriver)

  - Remove 2 hose clamps and hoses to suction regulator  Leave clamps on hoses (straight screwdriver)

  - Disconnect plug to Hobbs meter

  - Right cabin air at panel

- Outside Panel

  - Remove all but a couple screws

  - Install ramps

  - Remove final screws and slide partially out

  - Unplug Intercom

  - Remove Panel

REINSTALL INSTRUMENT PANEL

- Outside panel

  - Install ramps

  - Remove VM100 display (if installed)

  - Set panel on ramps

  - Plug in Intercom

  - Slide completely in place.  Watch Hobbs cable position

  - Install screws

  - Install gyro air filter Bolts to stiffner with long screw (3/8 socket, short straight screwdriver

- Under  Panel

  - Install right cabin air hose

  - Plug in Hobbs meter

  - Connect two air hoses to suction regulator (straight screwdriver)

  - Connect Pitot tube from encoder to rear VSI

  - Plug to fuel gages

  - Power plug to EC100

  - Flat cable to EC100

  - Connect DG power plug

  - Connect Static line -Red

  - Connect Pitot line - White

  - Install left cabin air hose

- Outside panel

  - Plug flat cable into Carb/outside air gage

  - Install VM100 flat panel  Plug flat cable before installation

  - Install Radio

  - Install Transponder

- Under Panel

  - Reconnect Battery