RV-10

N104CD

Checklist

Operational Checklist

RV-10  N104CD

AIRSPEEDS FOR SAFE OPERATION

Stall – Full Flaps (Vs0)	53 kts

Stall – No Flaps (Vs1)	62 kts

Best Glide (Vgl)	80 kts

Take-off

Lift-off	65 kts

Maximum Climb

Best Angle (Vx)	80 kts

Best Rate (Vy)	95 kts

Standard Climb	105 kts

Cruise Climb	115 kts

Structrual Cruising (Vno)	156 kts

Maneuvering Speed (Va)	125 kts

Approach Speed (Vproc)	90 kts

Flaps Extended (max) (Vfe)	87 kts

Landing Approach	75 kts

Balked Landing	105 kts

Maximum Demonstrated

Crosswind	25 kts

POWER PLANT DATA

LYCOMING	IO-540-D4A5

TYPE	6 CYL / FUEL INJECTED

HORSEPOWER	260

FUEL REQUIREMENT	100/100LL

PREFLIGHT INSPECTION

COCKPIT CHECK

Control Lock	REMOVE

All Switches	OFF

EXTERIOR CHECK

RIGHT FUSELAGE

Baggage Door	CHECK

Static Air Button	UNOBSTRUCTED

EMPENNAGE

Control Surfaces	CHECK

Tie Down	REMOVE

Position Light	CHECK

LEFT FUSELAGE

Static Air Button	UNOBSTRUCTED

All Antennas	CHECK

LEFT WING TRAILING EDGE

Flap	CHECK

Fuel Vent Line	UNOBSTRUCTED

Aileron	CHECK

Wing Tip	CHECK

Position Light	CHECK

LEFT WING LEADING EDGE

Pitot Tube	CHECK, (Remove Cover)

Landing Light	CHECK

Tie Down and Chocks	REMOVE

Stall Warning	CHECK for movement of vane

Fuel Tank	CHECK QUANTITY & SECURE

LEFT LANDING GEAR

Tire and Brake	CHECK

Fuel Sump	DRAIN

NOSE SECTION

Left Cowl	SECURE

Induction Air Intake	CLEAR

Propeller	CHECK, General Condition, Nicks,etc.

Tire and Nose Gear	CHECK

Engine Oil	CHECK Cap and Dipstick & SECURE

Right Cowl	SECURE

Fuel Strainer	DRAIN

Chocks	REMOVE

RIGHT LANDING GEAR

Fuel Sump	DRAIN

Tire and Brake	CHECK

RIGHT WING LEADING EDGE

Fuel Tank	CHECK QUANTITY & SECURE

Tie Down and Chocks	REMOVE

Wing Tip	CHECK

Position Light	CHECK

RIGHT WING TRAILING EDGE

Aileron	CHECK

Flap	CHECK

Fuel Tank Vent Line	UNOBSTRUCTED

BEFORE STARTING

Seats	POSITION AND LOCK

All Avionics	OFF

Circuit Breakers	IN

Flaps 	UP

Light Switches	AS REQUIRED

BATTERY & ALT Switch	ON

Aux. Batt Master Switch	ON

E-Bus Alt Feed	ON

Electronic Ignition Source	AUX

Fuel Boost Pump...ON (Check for operation, 	then OFF)

STARTING

Cold Start

Mixture	FULL RICH

Throttle	FAST IDLE position

Fuel Boost Pump	ON (max. of 3 sec. Then OFF)

Magneto/Start Switch	START position, 	RELEASE to both when engine fires

Hot Start

Mixture	IDLE CUT-OFF

Throttle	FAST IDLE -position

Magneto/Start Switch	ENGAGE

Mixture	ADVANCE MIXTURE SLOWLY when engine

starts firing regularly

Flooded Engine

Mixture	IDLE CUT-OFF

Throttle	FULL OPEN

Magneto/Starter Switch	ENGAGE

Mixture	ADVANCE MIXTURE SLOWLY when

engine starts firing regularly

Throttle	RETARD (to fast idle position)

Oil Pressure	YELLOW ARC (30 Seconds)

Warm-up	1000 to 1200 RPM

Engine Instruments	CHECK

Throttle	1500 RPM

Ammeter	CHECK

Throttle	IDLE

AFTER STARTING. AND BEFORE TAXI

Brakes	RELEASE AND CHECK

Avionics Master	ON

Nav Lights	ON

Strobe Lights	ON

Landing Lights	AS REQUIRED

E-Bus Alt Feed	OFF

Mixture	Lean aggressively to RPM Rise

Electronic Ignition Source	MAIN

# BEFORE TAKEOFF

Doors and Window	Secure

Seat Belts	CHECK

Flight Controls	CHECK FREE and CORRECT

Avionics	CHECK

Engine Instruments	CHECK

Flight Instruments	CHECK AND SET

Throttle	1700 RPM

Magnetos	CHECK at 1700 rpm

Throttle	1500 RPM

Ammeter	CHECK

Throttle	FAST IDLE

Elevator Trim	NOSE DOWN 2 BARS

Mixture	FULL RICH (or as required)

Engine Instruments	CHECK (Green Arc)

Flaps	Set to 0o

Set flaps one notch down from -3o reflex for takeoff

TAKEOFF

Takeoff & CLIMB	Full Throttle

(or 2700 RPM)

Rotate at 70 kts 	Climb at 105 kts

Airspeed 	105-110 kts

Set Climb at 25” / 2650RPM thru 1000’

Set Climb at 23” / 2500RPM for extended climbs

Flaps	Set to -3o reflex for Cruise

Fuel Boost	AS REQUIRED

CLIMB

Flaps	UP

Power	AS REQUIRED

Mixture	LEAN AS REQUIRED

Temperature	MONITOR

Fuel Boost	AS REQUIRED (off at 1000’ AGL)

# CRUISE

Power	SET AS DESIRED

Mixture	LEAN AS REQUIRED

DESCENT

Altimeter	SET

Power	AS REQUIRED

Mixture	ENRICH AS REQUIRED

BEFORE LANDING

Seat Belts	SECURE

Fuel Selector	SELECT FULLEST TANK

Mixture	FULL RICH (as required)

Landing and Taxi Lights	AS REQUIRED

Flaps	DOWN

Airspeed	ESTABLISH LANDING APPROACH

SPEED

BALKED LANDING

Mixture	FULL RICH (or as required)

Power	FULL THROTTLE or 2700 RPM

Airspeed……………..92 kts or until clear of 	obstacles

Flaps	UP

AFTER LANDING

Landing Lights	AS REQUIRED

Flaps	UP

Trim Tab	SET TO 0º

SHUTDOWN

Electrical and Avionics Equipment	OFF

Aux. Batt. Switch	OFF

E-Bus Alt Feed	OFF

Electronic Ignition Source	MAIN

Throttle	CLOSE

Mixture	IDLE CUT-OFF

Magneto/Start Switch	OFF, after engine stops

Aux. ALT Switch	OFF

BATTERY & ALT Switch	OFF

Control Lock	INSTALL

Aircraft	 TIE DOWN

Prior to engine shut-down the engine speed should be maintained between 1000 and 1200 RPM until the operating temperatures have stabilized. At this time the engine speed should be increased to approximately 1800 RPM for 15 to 20 seconds, then reduced to 1000 to 1200 RPM and shut-down immediately using the mixture control.

--- EMERGENCY PROCEDURES ---

EMERGENCY AIRSPEEDS

Emergency Descent	175 kts

Glide	80 kts

Emergency Landing Approach	75 kts

NOTE

Stall warning horn is inoperative when BATTERY & ALT

switch is turned off.

ENGINE FAILURE

DURING TAKE-OFF GROUND ROLL

Throttle	CLOSED

Braking	MAXIMUM

NOTE

Conduct the following procedures immediately if it appears certain that the airplane will run off the runway. (Otherwise conduct these procedures at the pilot's discretion.)

Fuel Selector Valve	OFF

BATTERY & ALT	OFF

Aux. ALT	OFF

Aux. Batt 	OFF

FUEL BOOST switches	OFF

Magneto/Start Switch 	OFF

AFTER LIFTOFF AND IN FLGHT

Landing straight ahead is usually advisable. if sufficient altitude is available for maneuvering, accomplish the following:

Fuel Selector Valve	SELECT OTHER TANK

(Check to feel detent and check visually)

Fuel Boost Pump	ON

Mixture	FULL RICH, then LEAN as required

Magnetos	CHECK LEFT/RIGHT, then BOTH

NOTE

The most probable cause of engine failure would be loss of fuel flow or improper functioning of the ignition system.

IF NO RESTART

Establish maximum glide	85 kts

Throttle	CLOSE

Fuel Selector Valve	OFF

Mixture	lDLE CUT-OFF

Magneto Start Switch	OFF

BATTERY & ALT, ALT switches	OFF

FUEL BOOST switch	OFF

When certain of reaching the selected landing site:

Airspeed	75 kts

Flaps	AS REQUIRED

BATTERY & ALT and ALT switches	OFF

ENGINE DISCREPANCY CHECKS

ROUGH RUNNING ENGINE

Mixture	FULL RICH, then LEAN as required

Magneto/Start Switch - CHECK LEFT,RIGHT,

then BOTH

LOSS OF ENGINE POWER

Fuel Flow or Fuel Pressure lndicator	CHECK

If fuel flow is abnormally low:

Mixture	FULL RICH

Fuel Boost Pump	ON (Lean as required)

Fuel Boost Pump	OFF if performance does

not improve in a few moments

Fuel Quantity Indicator	CHECK for fuel supply

in tank being used

If tank being used is empty:

Fuel Tank Selector Valve	SELECT OTHER

TANK (feel for detent and check visually)

AIR START PROCEDURE

Fuel Selector Valve	SELECT TANK MORE

NEARLY FULL (check to feel detent and check

visually)

Throttle	AS REQUIRED

Mixture	FULL RICH

Fuel Boost Pump	ON or OFF as required

Magneto/Start switch	BOTH

NOTE

When engine starts, adjust throttle, and mixture controls.

ENGINE FIRE IN FLIGHT

The ventilation controls must be closed to shut off all heating system outlets so that smoke and fumes will not enter the cabin. The control labeled CABIN AIR must be pulled aft to close. The control labeled DEFROST must be pushed forward to close. In the event of an engine fire, shut down the engine as follows and make a landing:

Fuel Selector Valve	OFF

Mixture	IDLE CUT-OFF

Throttle	CLOSE

Cabin Air Control	pull OFF

Defrost Valve	push OFF

BATTERY & ALT switch	OFF

Magneto/Start Switch	OFF

Do not attempt to restart engine

ENGINE FIRE ON THE GROUND

Fuel Selector Valve	OFF

Throttle	CLOSED

Mixture	IDLE CUT-OFF

BATTERY & ALT Switch	OFF

Magneto/Start Switch	OFF

Extinguish with Fire Extinguisher.

EMERGENCY DESCENT

Throttle	IDLE

Airspeed	ESTABLISH  /175 kts

(Smooth Air)

MAXIMUM GLIDE CONFIGURATION

Flaps	UP

Airspeed	85 kts

NOTE

Glide distance (in Zero Wind Conditions) is approximately

1.7 nautical miles (2 statute miles) per 1000 feet of altitude

above the terrain.

EMERGENCY LANDINGS

When assured of reaching the selected landing site, and

on final approach:

Airspeed	75 kts

Throttle	CLOSED

Fuel Selector Valve	OFF

Mixture	IDLE CUT-OFF

Magneto/Start Switch	OFF

Flaps	AS REQUIRED

BATTERY & ALT, ALT, and FUEL BOOST 	Switches OFF

SYSTEMS EMERGENCIES

ALTERNATOR-OUT PROCEDURE

NOTE

A failure of the alternator will place the entire electrical

operation of the airplane on the battery. Alternator

failure will be indicated by a discharging or fluctuating ammeter.

Aux. Batt Switch	ON

E-Bus Alt. Feed Switch	ON

Main ALT/BATT switch	OFF

All nonessential electrical loads	OFF

WARNING

Deactivation of the battery switch, alternator switch, or

alternator circuit breaker during flight is prohibited,

except as required by an actual emergency.

NOTE

For Airspeed Calibration and Altimeter Correction,

refer to PERFORMANCE section of the Pilots

Operating Handbook.