RV-10

N246RS

Checklist

Operational Checklist

RV-10  N246RS

AIRSPEEDS FOR SAFE OPERATION

Stall – Full Flaps (Vs0)	52 KIAS

Stall – No Flaps (Vs1)	60 KIAS

Best Glide (Vgl)	80 KIAS

Take-off

Lift-off	65 KIAS

Maximum Climb

Best Angle (Vx)	80 KIAS

Best Rate (Vy)	95 KIAS

Climb	105 KIAS

Cruise Climb	115 KIAS

Maneuvering Speed (Va)	125 KIAS

Max "G"............... ............................+ 3.8 to -1.9

Max Cruise 	155 KIAS

Do Not Exceed …………………..…...…..200 KIAS

Pattern Speed 	90 KIAS

Flaps Extended (1/2 and full) 	95 / 87 KIAS

Landing Approach (flaps)	70 - 75 KIAS

No Flaps .............................................

Go Around 	100 KIAS

Max Crosswind	25 KIAS

POWER PLANT DATA

LYCOMING	IO-540-D4A5

TYPE	6 CYL / FUEL INJECTED

HORSEPOWER	260

FUEL REQUIREMENT	100 LL

PREFLIGHT INSPECTION

COCKPIT CHECK

Control Lock	REMOVE

All Switches	OFF

EXTERIOR CHECK

RIGHT FUSELAGE

Static Air Port	CLEAR

TAIL

Control Surfaces	CHECK

Tie Down	REMOVE

Position Light	CHECK

LEFT FUSELAGE

Baggage Door	CLOSED

Static Air Port	CLEAR

All Antennas	CHECK

LEFT WING TRAILING EDGE

Flap	CHECK

Fuel Vent Line	UNOBSTRUCTED

Aileron	CHECK

Wing Tip	CHECK

Position Light	CHECK

LEFT WING LEADING EDGE

Pitot Tube	CHECK, (Remove Cover)

Fuel Tank	CHECK QUANTITY & SECURE

LEFT LANDING GEAR

Tire and Brake	CHECK

Chocks …………………………………REMOVE

Fuel Sump	DRAIN

NOSE SECTION

Left Cowl	SECURE

Induction Air Intake	CLEAR

Propeller	CHECK

Tire and Nose Gear	CHECK

Chocks ……………………….…………REMOVE

Engine Oil	 …CHECK - Cap & Dipstick SECURE

Right Cowl	SECURE

Fuel Strainer	DRAIN

RIGHT LANDING GEAR

Tire and Brake	CHECK

Chocks ………………………………… REMOVE

Fuel Sump	DRAIN

RIGHT WING LEADING EDGE

Fuel Tank	CHECK QUANTITY & SECURE

AOA Port …………………………….……CHECK

Wing Tip.	…CHECK

Position / Landing Light	CHECK

RIGHT WING TRAILING EDGE

Aileron	CHECK

Flap	CHECK

Fuel Tank Vent Line	UNOBSTRUCTED

PITOT HEAT………………………………....CHECK

BEFORE STARTING

Seats.	..POSITION AND LOCK

All Panel Switches 	OFF

Circuit Breakers	.IN

Flaps….. ………	.......CENTER POSITION

Batt 1 ……………………….…………………..ON

EIS  ……………….……………....CHECK ON

Bus Tie …………………..…...………………...ON

Annunciator Lights …….….……CHECK OPS

NOTE

Failure of EIS & Annunciator lights indicates bus tie relay inoperative.

Batt 2………………………….………………… ON

Bus Tie …………………………..……..……….OFF

Check Low Volts (2)..……….…..….….…...ON

Alternators (2) …………….…….…….…………ON

STARTING

Cold Start

Mixture	FULL RICH

Throttle	OPEN

Fuel Boost Pump	ON (1 - 2 sec. then OFF)

Mixture ........................................................OFF

Start Switch	START

Mixture .............................FULL RICH (on start)

Throttle .........................................IDLE SPEED

Engine Instruments

Oil Pressure………………...25 psi (30 seconds)

Fuel pressure………………………….12 psi min

Warm-up	1000 to 1200 RPM

Alternators (2) ……………………………CHECK

Hot Start

Mixture	OFF

Throttle	FAST IDLE

Start Switch	ENGAGE

Mixture	ADVANCE SLOWLY

(when engine starts running smoothly)

Flooded Engine

Mixture	OFF

Throttle	FULL OPEN

Starter Switch	ENGAGE

When engine starts

Throttle	IDLE

Mixture	ADVANCE SLOWLY

AFTER STARTING AND BEFORE TAXI

Avionics Master	ON

Nav & Strobe Lights	ON

Landing Lights….	.AS REQUIRED

Trim ………………...CHECK & SET 2 BARS DN

Flaps ……………………………………SET TO 0°

Brakes …………………………….………CHECK

# BEFORE TAKEOFF

Doors 	SECURE

Annunciators ………………………….…CHECK

Seat Belts	CHECK

Flight Controls	CHECK FREE & CORRECT

Autopilot Disconnect .............................CHECK

Avionics	CHECK

Pitot Heat ………………………………………ON

Flight Instruments	..CHECK AND SET

Throttle	1700 RPM

Magnetos	CHECK

Prop …………………………………. CHECK

Mixture ……………………………… CHECK

Engine Instruments	.CHECK

Alternators & Low Volt Lights	CHECK

Throttle	IDLE

Elevator Trim	CHECK 2 BARS DN

Flaps	CHECK 0o

Mixture	FULL RICH (or as required)

Fuel Pump ………………………………….…ON

TAKEOFF

Takeoff & CLIMB	Full Throttle

Rotate  	65 KIAS

Climb  	100-110 KIAS

Best Angle …………………………….80 KIAS

25” / 2650 RPM thru 1000’

23” / 2500 RPM for extended climbs

Flaps	 UP

CLIMB

Fuel Pump ……………………OFF (> 1,000AGL)

CHT …………………………………………< 435º

Oil Temp ……………………………..……..< 220º

CRUISE

Fuel Pump .......................................CHECK OFF

Power ............................................................SET

Mixture ........................................................LEAN

DESCENT

Altimeter	SET

Power	AS REQUIRED

Mixture ………………. RICHEN AS REQUIRED

Fuel Selector ……….SELECT FULLEST TANK

BEFORE LANDING

Seat Belts 	..  .SECURE

Fuel Pump ……………………………………..ON

Fuel Selector	CHECK / SET FULLEST TANK

Mixture	FULL RICH (as required)

Landing and Taxi Lights	AS REQUIRED

Flaps	DOWN

Airspeed …………………………....70 – 80 KIAS

GO-AROUND

Mixture	FULL RICH (or as required)

Prop ……………………………………MAX RPM

Power	FULL THROTTLE or 2700 RPM

Airspeed…………………………………..90 KIAS

Flaps	UP

AFTER LANDING

Fuel Pump ………………………………….   OFF

Landing Lights. 	.. AS REQUIRED

Flaps	FULL DOWN

Pitot Heat ……………………………………..OFF

Trim Tab	SET TO 0º

SHUTDOWN

Engine ............................IDLE TO COOL DOWN

Lights …………………….……………………. OFF

Avionics Master ……………………….………OFF

Alt 1 & 2 …………………….……….………….OFF

Prop ……………………………………. MAX RPM

Throttle ………………………………………...IDLE

Mixture ………………………………...FULL LEAN

All Switches…………………………………….OFF

Control Lock  …...	 ….  .. …INSTALL

Aircraft	 TIE DOWN

--- EMERGENCY PROCEDURES ---

ENGINE FIRE ON THE GROUND

MAGNETO / START SWITCH ...............START

FUEL SELECTOR	OFF

THROTTLE	OFF

MIXTURE	OFF

If Fire Continues

MAGNETO / START SWITCH	OFF

BATTERY & ALT Switch	OFF

Exit aircraft and extinguish with fire extinguisher

ENGINE FAILURE ON TAKEOFF (NOT AIRBORNE)

THROTTLE	CLOSED

BRAKES	MAXIMUM

If unable to land on runway

LAND STRAIGHT AHEAD (45° MAX TURN)

AIRSPEED ……………………….. 75 – 85 KIAS

PANEL SWITCHES …  OFF (PRIOR TO LAND)

START SWITCH ...	  OFF

FUEL SELECTOR .………………………….. OFF

MIXTURE .....................................................OFF

ENGINE FAILURE IN FLGHT

AIRSPEED ………………………… 75 – 85 KIAS

FUEL PUMP …………………………………..ON

FUEL SELECTOR	 SELECT OTHER TANK

(Check to feel detent and check visually)

MIXTURE  	. FULL RICH

THROTTLE ………………………….. FAST IDLE

START SWITCH..	BOTH (START if prop stopped)

NOTE

When engine starts, adjust throttle & mixture controls

The most probable cause of engine failure would

be loss of fuel flow or improper functioning of the ignition system.

IF NO RESTART

MAYDAY CALL & SQUAWK 7700

MAX RANGE GLIDE SPEED	80 KIAS

THROTTLE	IDLE

FUEL SELECTOR	OFF

MIXTURE	OFF

MAG SWITCH	OFF

FUEL PUMP	OFF

When certain of reaching the selected landing site:

AIRSPEED	75 kts

FLAPS	AS REQUIRED

ALL PANEL SWITCHES 	OFF

EMERGENCY AIRSPEEDS

EMERG DESCENT	175 kts

BEST GLIDE	80 kts

EMERG APPROACH	75 kts

ENGINE DISCREPANCY CHECKS

ROUGH RUNNING ENGINE

FUEL PUMP ………………………………….ON

MIXTURE …	FULL RICH, then LEAN as req’d

MAG SWITCH……….. CHECK LEFT, RIGHT,

then BOTH

LOSS OF ENGINE POWER

FUEL FLOW / PRESSURE	CHECK

If fuel flow is abnormally low:

MIXTURE	FULL RICH

FUEL PUMP	ON (Lean as required)

FUEL QUANTITY	CHECK TANK

If tank being used is empty:

FUEL SELECTOR ……SELECT OTHER TANK

(feel for detent and check visually)

ENGINE FIRE IN FLIGHT

FUEL SELECTOR	OFF

MIXTURE	OFF

THROTTLE	IDLE

CABIN AIR CONTROL	.PULL OFF

ALTERNATOR SWITCHES (2)…………….OFF

BATTERY SWITCHES (2)	OFF

MAG SWITCH	OFF

Do not attempt to restart engine

WARNING

The ventilation controls must be closed to shut off all heating system outlets so that smoke and fumes will not enter the cabin.

SPIN RECOVERY

CONTROLS .............................................RELEASE

If Spin Continues

THROTTLE ...............................................IDLE

STICK ..................................................CENTER

RUDDER .......................RULL OPPOSITE SPIN

RECOVER FROM DIVE

EMERGENCY DESCENT

Throttle	IDLE

Airspeed	ESTABLISH  175 KIAS

(slower for rough air)

MAXIMUM GLIDE CONFIGURATION

Flaps	UP

Airspeed	80 KIAS

## NOTE

Glide distance (in Zero Wind Conditions) is approximately

1.7 nautical miles per 1000 feet of altitude

EMERGENCY LANDING

AIRSPEED	75 KIAS

THROTTLE ………………………………… IDLE

FUEL SELECTOR..………………………….OFF

MIXTURE ………………………………..….. OFF

FLAPS	AS REQUIRED

ALL PANEL SWITCHES 	OFF

SEAT BELTS ......................................SECURE

DOORS .................UNLATCH (prior to landing)

#### ELECTRICAL MALFUNCTIONS

#### LOW VOLT LIGHT…………….CHECK VOLT OUTPUTS

###### (CHECK ALT 1 on Chelton screen, ALT 2 on GRT EIS screen)

###### IF <12.5v & DECR SLOWLY…………...BUS TIE ON

IF NO IMMEDIATE RISE > 13v…….…..BUS TIE OFF

### CAUTION

If voltage does not immediately rise back to over 13v, this indicates serious battery or other electrical problems on the bus other than simple alternator or voltage regulator failure.  Turn applicable battery bus OFF.

LOSS OF SINGLE BUSS - TURN THAT BUS MASTER OFF

CAUTION

DO NOT activate bus tie switch.  If the battery has shorted, this will result in the good bus being rendered inoperative.

NOTE

Bus 2 alternator cannot power power all loads on bus 1.  Turn off high amp equip such as pitot heat, nav lights, landing light and strobes.

ELECTRICAL BUS DISTRIBUTION

BUS 1                                           BUS 2

Alt 1                                               Alt 2

Low Volt Sense 1                          Low Volt Sense 2

Fuel Pump                                     EIS

Nav, Taxi, Strobe lights                 Inst Lights

Pitot Heat                                       B/U Attitude

AHRS                                            Annunciators

Trim                                               GPS

Transponder                                  Auto Pilot

Comm 1                                         Comm 2

Top MFD                                        Bottom MFD

Intercom

AVIONICS MASTER SWITCH

Bus 1                                             Bus 2

Com 1                                            Com 2

Nav 1                                              Nav 2

Transponder                                   Audio Panel

AHRS                                             Autopilot

MFD 1 (Top)                                   MFD 2 (Bottom)

New Engine Break In

CHT - 435 MAX    OIL TEMP - < 230

1. Avoid prolonged ground runs

2. Full power for take-off

3. Full rich mixture except high alt to smooth roughness

4. 70-75% power cruise, remain below 7,000ft

5. Power on descents

6. Mineral oil 1st 25 hrs.  No synthetic blends till 100hrs

7. Keep temps within manufacturer specs