|  | Emergency |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Engine Failure |  |  |  |  |
|  | Airspeed - Best glide |  |  | 80 KIAS |  |
|  | Pick landing location |  |  | Into wind |  |
|  | Mixture |  |  | Full In (Rich) |  |
|  | Alternate Air |  |  | Pull and leave it Pulled |  |
|  | Magnetos |  |  | Check |  |
|  | Fuel Selector |  |  | Fullest Tank |  |
|  | Ignition |  |  | Both or Start |  |
|  | Flaps |  |  | As Required |  |
|  | Emergency Frequency |  |  | 121.5 |  |
|  | Emergency Squawk Code |  |  | 7700 |  |
|  | Master switch |  |  | Off |  |
|  | Cabin Doors |  |  | Unlatch |  |
|  | Touchdown |  |  | Tail Low |  |
|  | Emergency Descent |  |  |  |  |
|  | Throttle |  |  | Idle |  |
|  | Mixture |  |  | Full In (Rich) |  |
|  | Propeller |  |  | Full In (High RPM) |  |
|  | Flaps |  |  | Full |  |
|  | Bank at 45 Deg |  |  |  |  |
|  | Descend at V(FE) |  |  | 87 KIAS |  |
|  | Fire during Start Up |  |  |  |  |
|  | Starter |  |  | Continue Cranking |  |
|  | Mixture |  |  | Full Out (Lean) |  |
|  | Throttle |  |  | Full In (Open) |  |
|  | Fuel Selector |  |  | Off |  |
|  | Fire In Flight |  |  |  |  |
|  | Fuel Selector |  |  | Off |  |
|  | Throttle |  |  | Full Out (Idle) |  |
|  | Mixture |  |  | Full Out (Lean) |  |
|  | Fire Extinguisher |  |  | Undo Clasp and pull Pin |  |
|  | Fire - Electrical |  |  |  |  |
|  | Master Switch |  |  | Off |  |
|  | Vents |  |  | Open |  |
|  | Cabin Heat |  |  | Off |  |
|  | Radio failure / Light Signals |  |  |  |  |
|  | If Tower Controlled, Squawk 7600, Pattern + 500 ft. |  |  |  |  |
|  | Steady Green |  |  | Clear to Land / Takeoff |  |
|  | Flashing Green |  |  | Return to Land |  |
|  | Steady Red |  |  | Giveway to Aircraft |  |
|  | Flashing Red |  |  | Unsafe / Clear Runway |  |
|  | Flashing White |  |  | Return to Starting Point |  |
|  | Flashing Red / Green |  |  | Warning - Caution |  |
|  | N519RV (Van’s RV-10) Airspeeds (KIAS) |  |  |  |  |
|  | Vs0 58 |  |  | Vfe 87 |  |
|  | Vs1 63 |  |  | Va 107 |  |
|  | Vx 80 |  |  | Vno 160 |  |
|  | Vy 80 |  |  | Vne 200 |  |
|  | Kansas City Approach (Class B) Frequencies |  |  |  |  |
|  | 118.9 - South of LWC Apt. |  |  | 124.7 – West of MCI Rwy 01/19 |  |
|  | 119.0 – East of MCI Rwy 01/19 |  |  |  |  |
|  | 1-800-WX-BRIEF |  |  | 1-800-992-7433 |  |
| N519RV (Van’s RV-10 ) - Preflight |  |  |  |  |  |
| Cockpit Pre-Flight Inspection |  |  |  |  |  |
| Aircraft Docs (ARROW) |  |  | Check |  |  |
| Weight & Balance |  |  | Check |  |  |
| Ignition |  |  | Off |  |  |
| All Switches |  |  | Off |  |  |
| Master Switch |  |  | On (Battery Position) |  |  |
| Avionics Main Switch |  |  | On |  |  |
| Left and Right Fuel Gauge |  |  | Check |  |  |
| Fuel Remaining on EMS |  |  | Adjust if Fuel Added |  |  |
| Trip Timer on EMS |  |  | Reset when Fully Fueled |  |  |
| Flaps |  |  | Extend |  |  |
| Master & Avionics Main Switch |  |  | Off |  |  |
| Pitot Heat (if required) |  |  | Test for Heat |  |  |
| Windows |  |  | Clean |  |  |
| Fuselage and Empennage Pre-Flight Inspection |  |  |  |  |  |
| Baggage Door and Lock |  |  | Check |  |  |
| Static Ports |  |  | Check |  |  |
| Control Surfaces |  |  | Free and Secure |  |  |
| Tie Down |  |  | Disconnect |  |  |
| Trim Tab |  |  | Check security |  |  |
| Antennas |  |  | Check security |  |  |
| Nav/Strobe Lights |  |  | Check Lens and Lamps |  |  |
| Rudder/Trim Cables |  |  | Check |  |  |
| Right Wing Pre-Flight Inspection |  |  |  |  |  |
| Flaps |  |  | Secure |  |  |
| Aileron |  |  | Free and Secure |  |  |
| Nav/Landing/Strobe Lights |  |  | Check Lens and Lamps |  |  |
| Tie Down |  |  | Disconnect and Remove |  |  |
| Fuel Quantity |  |  | Visually Check |  |  |
| Fuel Filler Cap |  |  | Secure |  |  |
| Main Wheel |  |  | Condition and Inflated to 40 PSI |  |  |
| Brakes |  |  | Check Pads and Lines for Leaks |  |  |
| Fuel Tank Sump |  |  | Sample |  |  |
| Fuel Tank Vent |  |  | Check |  |  |
| Firewall Forward Pre-Flight Inspection |  |  |  |  |  |
| Nose Wheel |  |  | Condition and Inflated to 38 PSI |  |  |
| Prop & Spinner |  |  | Check |  |  |
| Alternator Belt |  |  | Check |  |  |
| Air Intake, Filter & Prop Cable |  |  | Check |  |  |
| Engine Oil Level |  |  | Check (Run at 9 Qt, Add at 8 Qt) |  |  |
| Hinge Pins |  |  | Check and Secure |  |  |
| Left Wing Pre-Flight Inspection |  |  |  |  |  |
| Main Wheel |  |  | Condition and Inflated to 40 PSI |  |  |
| Brakes |  |  | Check Pads and Lines for Leaks |  |  |
| Fuel Tank Sump |  |  | Sample |  |  |
| Fuel Tank Vent |  |  | Check |  |  |
| Fuel Quantity |  |  | Check |  |  |
| Fuel Filler Cap |  |  | Secure |  |  |
| Pitot Tube |  |  | Uncover and Check |  |  |
| Tie Down |  |  | Disconnect and Remove |  |  |
| Nav/Taxi/Strobe Lights |  |  | Check Lens and Lamps |  |  |
| Aileron |  |  | Free and Secure |  |  |
| Flaps |  |  | Secure |  |  |
|  | Ground Procedures |  |  |  |  |
|  | Before Starting Engine |  |  |  |  |
|  | Preflight Inspection | Complete |  |  |  |
|  | Engine Controls | Free |  |  |  |
|  | Fuel Selector | Fullest/Desired Tank |  |  |  |
|  | Seats Belts | On/Adjusted |  |  |  |
|  | Main & Aux Avionics Switches | Off |  |  |  |
|  | Parking Brake & Brakes | Parking Brake (Release), Brakes (Set) |  |  |  |
|  | Engine Starting and Run Up |  |  |  |  |
|  | Master Switch | On (Battery Position) |  |  |  |
|  | Flaps | Full Up |  |  |  |
|  | Alternate Air | Full In (Don’t pull on the Ground) |  |  |  |
|  | Propeller | Full In (High RPM) |  |  |  |
|  | Cold Start |  |  |  |  |
|  | Mixture | Full In (Rich) |  |  |  |
|  | Throttle | 1/8 Inch |  |  |  |
|  | Boost Pump | Run till you hear the Pressure Build |  |  |  |
|  | Hot Start |  |  |  |  |
|  | Mixture | Full Out until it fires then Full In |  |  |  |
|  | Throttle | 1/8 Inch |  |  |  |
|  | Boost Pump | On once Mixture Full In, then Off |  |  |  |
|  | Propeller Area | Clear |  |  |  |
|  | Ignition | Start |  |  |  |
|  | Throttle | Adjust to 1000 - 1200 RPM |  |  |  |
|  | Master Switch | On (Alternator Position) |  |  |  |
|  | Main & Aux Avionics Switch | On |  |  |  |
|  | Oil Pressure | Check |  |  |  |
|  | Ammeter | Charging (Positive Value) |  |  |  |
|  | Voltmeter | +14 Volts |  |  |  |
|  | EFIS Altimeter and Altimeter | Set for Field Elevation (+/- 75 Ft) |  |  |  |
|  | ATIS/AWOS/ASOS | Obtain |  |  |  |
|  | Cabin Doors | Closed & Locked (Check Door/Frame Gap) |  |  |  |
|  | Seats Belts | Adjust & Lock |  |  |  |
|  | Flight Controls | Free & Correct |  |  |  |
|  | Elevator Trim | Takeoff Position |  |  |  |
|  | Throttle Setting | 2000 rpm |  |  |  |
|  | Magnetos | Check |  |  |  |
|  | Propeller | Cycle (500 rpm drop) |  |  |  |
|  | Engine Instruments | Check |  |  |  |
|  | Mixture | Lean (Out 1”) for Taxi |  |  |  |
|  | Radio/Nav Equipment | Setup desired Freq and Nav Points |  |  |  |
|  | Transponder | VFR(1200) or Squawk Code |  |  |  |
|  | Nav Lights | On for Night Flying |  |  |  |
|  | Strobe Lights | On |  |  |  |
|  | Taxi/Landing Light | On for Night Takeoff |  |  |  |
|  | Wig-Wag Lights | On (Off if Taxi/Landing Light is Used) |  |  |  |
|  | Pitot Heat | As Required |  |  |  |
|  | Before Takeoff |  |  |  |  |
|  | Fuel Selector | Fullest/Desired Tank |  |  |  |
|  | Boost Pump | On (Off once 1000 AGL) |  |  |  |
|  | Mixture | Full In (Rich) |  |  |  |
|  | In-Flight / Post Flight |  |  |  |  |
|  | Takeoff |  |  |  |  |
|  | Flaps |  |  |  | Full Up |
|  | Power |  |  |  | Full Throttle (Prop Full In) |
|  | Elevator |  |  |  | Lift at 60 KIAS |
|  | Climb ( > 1000 Feet AGL) |  |  |  | 110 KIAS (25” @ 2500 RPM) |
|  | Cruise |  |  |  |  |
|  | Set Desired Power |  |  |  | 20 to 24.5 Squared |
|  | Mixture |  |  |  | 25 – 75 lean of Peak if < 75% Pwr |
|  | EGT |  |  |  | < 1500 degrees F |
|  | CHT |  |  |  | < 425 degrees F |
|  | Engine Instruments |  |  |  | Scan |
|  | Before Landing |  |  |  |  |
|  | Mixture |  |  |  | Full In (Rich) |
|  | Boost Pump |  |  |  | On |
|  | Propeller |  |  |  | Full In (High RPM) if < 80 KIAS |
|  | Flaps (below 87 KIAS) |  |  |  | Half or Full (As Desired) |
|  | Approach Speed |  |  |  | 70 - 80 KIAS (Load Dependent) |
|  | After Landing |  |  |  |  |
|  | Flaps |  |  |  | Full Up |
|  | Mixture |  |  |  | Lean 1" for Taxi |
|  | Boost Pump |  |  |  | Off |
|  | Transponder |  |  |  | Standby (Check) |
|  | All Lights |  |  |  | As Required |
|  | Shutdown / Parking |  |  |  |  |
|  | Throttle |  |  |  | 1000 RPM |
|  | Flight Time |  |  |  | Record from EMS |
|  | All Switches |  |  |  | Off (from Right to Left) |
|  | Mixture |  |  |  | Full Lean |
|  | Ignition / Magnetos |  |  |  | Off (Remove Key) |
|  | CLOSE FLIGHT PLAN |  |  |  | CLOSE FLIGHT PLAN |
|  | Securing the Aircraft |  |  |  |  |
|  | Parking Brake |  |  |  | Set |
|  | Stick Controls |  |  |  | Secure with Seat Belt |
|  | Rudder |  |  |  | Secure |
|  | Cabin Doors |  |  |  | Secure |
|  | Pitot Tube |  |  |  | Cover |
|  | Tie Downs |  |  |  | Install and Secure |
|  | Chocks |  |  |  | Install |
|  | Nearby Airports |  |  |  |  |
|  | Johnson Cnty Exec - OJC |  |  |  | New Century – IXD |
|  | 18-36 1096 ft. |  |  |  | 17-35 & 04-22 1087 ft. |
|  | Tower 126.0 |  |  |  | Tower 133.0 |
|  | Ground 121.6 |  |  |  | Ground 124.3 |
|  | VOR 113.0 |  |  |  | AWOS 135.325 |
|  | ATIS 119.35 |  |  |  |  |
|  | Gardner - K34 |  |  |  | Miami Co. - K81 |
|  | 03-21, 08-26, & 17-35 |  |  |  | 03-21 940 ft. |
|  | Elev 1042 ft. |  |  |  | CTAF 122.8 |
|  | CTAF 122.8 |  |  |  |  |